Answers for questions

- When SNR increases, BER decreases exponentially. The higher SNR, the stronger signal power, and the signal levels will be received more accurately.

- No. According to the panel, at SNR = 15 dB, the expected BER is approximately larger than 10^-5. At SNR = 11 dB, the BER is approximately 10^-4, and BER decreases exponentially.

- As the filter bandwidth decreases, BER increases. The smaller bandwidth is closer to the Nyquist rate (10 GHz for 20 Gbits/s), there will be Intersymbol Interference.

- Signal levels: [-3/√5, -1/√5, 1/√5, 3/√5]

Threshold 1 = (-3/√5 + -1/√5)/2 = -2/√5

Threshold 2 = (-1/√5 + 1/√5)/2 = 0

Threshold 3 = (1/√5 + 3/√5)/2 = 2/√5

Thus, the detection thresholds are [-2/√5, 0, 2/√5]

- BER will increase. Signal may be disabled to distinguish.

%% Lab 2: Digital Transmission Simulations

clear all; close all; clc;

fprintf('=== Lab 2: Digital Transmission Simulations - Complete Version ===\n');

%% ==================== Step 1: Generate random bit vector v ====================

fprintf('\n=== Step 1: Generate random bit vector v ===\n');

num_bits = 100000;

v = 2*randi([0 1], 1, num_bits) - 1;

fprintf('Generated %d random bits (1 and -1)\n', num_bits);

%% ==================== Step 2: Generate digital waveform s ====================

fprintf('\n=== Step 2: Generate digital waveform s ===\n');

samples_per_bit = 10;

bit_period = 50e-12; % 50 ps

sample_interval = bit_period / samples_per_bit; % 5 ps

% Generate NRZ waveform using proper method

s = zeros(1, num_bits * samples_per_bit);

for i = 1:num_bits

start_idx = (i-1)*samples_per_bit + 1;

end_idx = i*samples_per_bit;

s(start_idx:end_idx) = v(i); % Each bit repeated samples_per_bit times

end

% Verify implementation

fprintf('Length verification:\n');

fprintf('  Vector v length: %d bits\n', length(v));

fprintf('  Waveform s length: %d samples\n', length(s));

fprintf('  Length ratio: s/v = %d/%d = %.1f\n', length(s), length(v), length(s)/length(v));

if length(s) == length(v) * samples_per_bit

fprintf('  ✓ Success: s length is %d times v length\n', samples_per_bit);

else

fprintf('  ✗ Error: s length is not %d times v length\n', samples_per_bit);

end

% Calculate signal power

Ps = mean(s.^2);

fprintf('Signal power Ps = %.4f\n', Ps);

%% ==================== Step 3-6: Binary NRZ BER Simulation ====================

fprintf('\n=== Steps 3-6: Binary NRZ BER Simulation ===\n');

SNR_dB = [5, 7, 9, 11, 13, 15, 17];

BER_binary = zeros(size(SNR_dB));

% Define sampling points (middle of each bit)

sample_points = round(samples_per_bit/2) : samples_per_bit : length(s);

for i = 1:length(SNR_dB)

% Convert SNR from dB to linear scale

SNR_linear = 10^(SNR_dB(i)/10);

% Calculate noise variance and standard deviation

Var_noise = Ps / SNR_linear;

sigma_noise = sqrt(Var_noise);

% Add Gaussian noise to the signal

y = s + sigma_noise * randn(size(s));

% Sample at middle of each bit

received_samples = y(sample_points);

% Make bit decisions (threshold at 0)

decisions = sign(received_samples);

% Calculate BER

errors = sum(decisions ~= v);

BER_binary(i) = errors / num_bits;

fprintf('SNR = %2d dB: Error bits = %d, BER = %.2e\n', ...

SNR_dB(i), errors, BER_binary(i));

end

% Plot binary NRZ results

figure('Position', [100, 100, 800, 600]);

semilogy(SNR_dB, BER_binary, 'bo-', 'LineWidth', 2, 'MarkerSize', 8);

grid on;

xlabel('SNR (dB)');

ylabel('Bit Error Rate (BER)');

title('Binary NRZ Signal - BER vs SNR');

legend('Simulation Results', 'Location', 'southwest');

%% ==================== Step 7: Channel with Low-Pass Filter (using fft_PolyU/ifft_PolyU) ====================

fprintf('\n=== Step 7: Channel with Low-Pass Filter (using fft_PolyU/ifft_PolyU) ===\n');

bandwidths = [13e9, 10e9, 9e9]; % 13 GHz, 10 GHz, 9 GHz

BER_filtered = zeros(length(bandwidths), length(SNR_dB));

% Create time vector

t = (0:length(s)-1) * sample_interval;

fprintf('System parameters: Sampling frequency = %.0f GHz\n', 1/sample_interval/1e9);

for bw_idx = 1:length(bandwidths)

bw = bandwidths(bw_idx);

fprintf('\n--- Filter bandwidth: %.0f GHz ---\n', bw/1e9);

% Use fft_PolyU to transform to frequency domain

[omega, spectrum] = fft_PolyU(t, s);

% Convert angular frequency to Hz and create ideal low-pass filter

freq_Hz = omega / (2*pi); % Convert rad/s to Hz

H = double(abs(freq_Hz) <= bw);

% Apply filter in frequency domain

filtered_spectrum = spectrum .* H;

% Use ifft_PolyU to transform back to time domain

[t_filtered, u] = ifft_PolyU(omega, filtered_spectrum);

% Ensure the signal is real (remove small imaginary parts)

u = real(u);

% Calculate filtered signal power

Ps_filtered = mean(u.^2);

fprintf('Filtered signal power: %.4f\n', Ps_filtered);

for i = 1:length(SNR_dB)

% Calculate noise parameters

SNR_linear = 10^(SNR_dB(i)/10);

Var_noise = Ps_filtered / SNR_linear;

sigma_noise = sqrt(Var_noise);

% Add noise to filtered signal

y = u + sigma_noise * randn(size(u));

% Sample and make decisions

received_samples = y(sample_points);

decisions = sign(received_samples);

% Calculate BER

errors = sum(decisions ~= v);

BER_filtered(bw_idx, i) = errors / num_bits;

fprintf('  SNR = %2d dB: BER = %.2e\n', SNR_dB(i), BER_filtered(bw_idx, i));

end

end

% Plot filtered results

figure('Position', [100, 100, 800, 600]);

colors = ['r', 'g', 'm'];

legend_names = {};

for bw_idx = 1:length(bandwidths)

semilogy(SNR_dB, BER_filtered(bw_idx,:), [colors(bw_idx) 'o-'], ...

'LineWidth', 2, 'MarkerSize', 8);

hold on;

legend_names{end+1} = sprintf('%.0f GHz', bandwidths(bw_idx)/1e9);

end

grid on;

xlabel('SNR (dB)');

ylabel('Bit Error Rate (BER)');

title('Binary Signals with Different Filter Bandwidths - BER vs SNR');

legend(legend_names, 'Location', 'southwest');

%% ==================== Step 8: PAM-4 Signal Simulation ====================

fprintf('\n=== Step 8: PAM-4 Signal Simulation ===\n');

% Define PAM-4 levels (normalized for unit average power)

pam4_levels = [-3/sqrt(5), -1/sqrt(5), 1/sqrt(5), 3/sqrt(5)];

fprintf('PAM-4 levels: [%.4f, %.4f, %.4f, %.4f]\n', pam4_levels);

% Generate PAM-4 symbols

symbol_indices = randi([1, 4], 1, num_bits);

v_pam4 = pam4_levels(symbol_indices);

% Verify average power

Ps_pam4 = mean(v_pam4.^2);

fprintf('PAM-4 average power: %.4f (target: 1.0000)\n', Ps_pam4);

% Generate PAM-4 waveform

s_pam4 = zeros(1, num_bits * samples_per_bit);

for i = 1:num_bits

start_idx = (i-1)*samples_per_bit + 1;

end_idx = i*samples_per_bit;

s_pam4(start_idx:end_idx) = v_pam4(i);

end

% Define detection thresholds (midpoints between levels)

thresholds = [-2/sqrt(5), 0, 2/sqrt(5)];

fprintf('PAM-4 detection thresholds: %.4f, %.4f, %.4f\n', thresholds);

BER_pam4 = zeros(size(SNR_dB));

for i = 1:length(SNR_dB)

% Calculate noise parameters

SNR_linear = 10^(SNR_dB(i)/10);

Var_noise = Ps_pam4 / SNR_linear;

sigma_noise = sqrt(Var_noise);

% Add noise to PAM-4 signal

y = s_pam4 + sigma_noise * randn(size(s_pam4));

% Sample at middle of each symbol

received_samples = y(sample_points);

% PAM-4 symbol decision

decisions_pam4 = zeros(size(received_samples));

for j = 1:length(received_samples)

if received_samples(j) < thresholds(1)

decisions_pam4(j) = 1;

elseif received_samples(j) < thresholds(2)

decisions_pam4(j) = 2;

elseif received_samples(j) < thresholds(3)

decisions_pam4(j) = 3;

else

decisions_pam4(j) = 4;

end

end

% Calculate Symbol Error Rate and Bit Error Rate

symbol_errors = sum(decisions_pam4 ~= symbol_indices);

SER = symbol_errors / num_bits;

BER_pam4(i) = 0.5 * SER; % Assuming Gray coding

fprintf('SNR = %2d dB: Symbol errors = %d, SER = %.2e, BER = %.2e\n', ...

SNR_dB(i), symbol_errors, SER, BER_pam4(i));

end

% Plot PAM-4 results

figure('Position', [100, 100, 800, 600]);

semilogy(SNR_dB, BER_pam4, 'ko-', 'LineWidth', 2, 'MarkerSize', 8);

grid on;

xlabel('SNR (dB)');

ylabel('Bit Error Rate (BER)');

title('PAM-4 Signal - BER vs SNR');

legend('PAM-4', 'Location', 'southwest');

%% ==================== Comprehensive Comparison ====================

% Plot comprehensive comparison

figure('Position', [100, 100, 1000, 600]);

semilogy(SNR_dB, BER_binary, 'bo-', 'LineWidth', 2, 'MarkerSize', 8);

hold on;

for bw_idx = 1:length(bandwidths)

semilogy(SNR_dB, BER_filtered(bw_idx,:), [colors(bw_idx) 'o-'], ...

'LineWidth', 2, 'MarkerSize', 8);

end

semilogy(SNR_dB, BER_pam4, 'ko-', 'LineWidth', 2, 'MarkerSize', 8);

grid on;

xlabel('SNR (dB)');

ylabel('Bit Error Rate (BER)');

title('Comparison of All Signal Formats - BER vs SNR');

legend_names = {'Binary NRZ'};

for bw_idx = 1:length(bandwidths)

legend_names{end+1} = sprintf('Binary + %.0fGHz filter', bandwidths(bw_idx)/1e9);

end

legend_names{end+1} = 'PAM-4';

legend(legend_names, 'Location', 'southwest');

%% ==================== Theoretical vs Simulated BER Comparison ====================

% Calculate theoretical BER for binary NRZ

SNR_range = 0:0.5:20;

theory_BER = 0.5 * erfc(sqrt(10.^(SNR_range/10)));

figure('Position', [100, 100, 800, 600]);

semilogy(SNR_range, theory_BER, 'r-', 'LineWidth', 2, 'DisplayName', 'Theoretical BER');

hold on;

semilogy(SNR_dB, BER_binary, 'bo-', 'LineWidth', 2, 'MarkerSize', 8, 'DisplayName', 'Simulated BER');

grid on;

xlabel('SNR (dB)');

ylabel('Bit Error Rate (BER)');

title('Theoretical vs Simulated BER - Binary NRZ Signal');

legend('Location', 'southwest');

fprintf('\n=== All steps completed successfully ===\n');

fprintf('Generated 5 figures:\n');

fprintf('  1. Binary NRZ Signal - BER vs SNR\n');

fprintf('  2. Binary Signals with Different Filter Bandwidths - BER vs SNR\n');

fprintf('  3. PAM-4 Signal - BER vs SNR\n');

fprintf('  4. Comparison of All Signal Formats - BER vs SNR\n');

fprintf('  5. Theoretical vs Simulated BER - Binary NRZ Signal\n');